# Vending Machine FSM — State Diagram

Module: vending_machine   |   States: 4   |   Encoding: 2-bit enum (state_t)   |   Clock: posedge clk, Reset: negedge rst_n (async)

## 1. State Overview

| State | Encoding | Description |
| --- | --- | --- |
| IDLE | 2'b00 | Waiting for user input. Accepts pro_chose, check_out, or tak_bac. |
| DIS_PRO | 2'b01 | Product browse mode. User views items (displaying=pro_detail), adds them to cart, navigates to checkout, or returns with back/timer. |
| PRO_CHEC | 2'b10 | Checkout review. User can add money, confirm payment (cash or card), or cancel. On success: dispenses products and returns change. |
| GIV_BAC | 2'b11 | Money return. Dispenses remaining balance back to user. Automatically returns to IDLE on the next clock cycle. |

## 2. ASCII State Diagram

rst_n='0' (async reset to IDLE)

¦

v

┌──────────────────────────────────┐

│ IDLE │ <─── GIV_BAC (auto)

└──────────────────────────────────┘

¦ ¦ ¦

pro_chose check_out tak_bac

¦ ¦ ¦

v v v

┌─────────┐ ┌─────────┐ ┌─────────┐

│ DIS_PRO │ │PRO_CHEC │ │ GIV_BAC │

└─────────┘ └─────────┘ └─────────┘

¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦

¦ ¦ +------+ ¦ ¦ +------+ (auto -> IDLE)

¦ ¦ check_out ¦ ¦ back/timer

¦ ¦ ¦ ¦

¦ +--[self] ¦ +--[self] add_mon

¦ add item ¦ balance += mon_add

¦ ¦

back/timer conf + afford

-> IDLE OR pay_card

-> IDLE

dispense, trans_done=1

conf + !afford

+ !pay_card

-> [self]

error=1

## 3. Complete State Transition Table

| Current State | Condition / Input | Next State | Actions / Outputs |
| --- | --- | --- | --- |
| IDLE | tak_bac | GIV_BAC | mo_out = balance (or mo_lef) |
| IDLE | check_out | PRO_CHEC | balance = mo_lef |
| IDLE | pro_chose | DIS_PRO | balance = mo_lef |
| GIV_BAC | (always, next clock) | IDLE | balance=0, payment=0, pro_count=0, trans_completed=1 |
| DIS_PRO | back / timer | IDLE | — |
| DIS_PRO | check_out | PRO_CHEC | — |
| DIS_PRO | add (pro_count < 11) | DIS_PRO | cart[pro_count]=pro_detail, payment+=price[pro_detail], pro_count++ |
| PRO_CHEC | conf + (afford OR pay_card) | IDLE | pro_out=cart[], balance-=payment, trans_done=1, mo_left=balance-payment |
| PRO_CHEC | conf + !afford + !pay_card | PRO_CHEC | error=1, shortfall=payment-balance |
| PRO_CHEC | add_mon | PRO_CHEC | balance += mon_add |
| PRO_CHEC | back / timer | IDLE | trans_done=0, error=0, shortfall=0 |

## 4. Key Signals Reference

| Signal | Type | Description |
| --- | --- | --- |
| pro_chose | Input | Pulse: enter product browse mode → IDLE to DIS_PRO |
| check_out | Input | Pulse: go to checkout → IDLE/DIS_PRO to PRO_CHEC |
| tak_bac | Input | Pulse: return money → IDLE to GIV_BAC |
| add | Input | Pulse: add current pro_detail to cart (DIS_PRO self-loop) |
| back | Input | Pulse: cancel / go back → DIS_PRO or PRO_CHEC to IDLE |
| timer | Input | Pulse: inactivity timeout → same as back |
| add_mon | Input | Pulse: add mon_add to balance while in PRO_CHEC |
| conf | Input | Pulse: confirm payment in PRO_CHEC |
| pay_card | Input | High: card approved — overrides cash balance check |
| afford | Internal | Combinational: balance >= payment |
| trans_done | Output | High for one cycle when purchase completes successfully |
| error | Output | High when conf attempted but balance < payment and no card |
| shortfall | Output | Amount still owed: payment - balance |
| mo_out | Output | Change dispensed to user on GIV_BAC |
| mo_left | Output | Remaining balance after successful purchase |
| displaying | Output | Product index shown on screen during DIS_PRO |
| pro_out[] | Output | Array of dispensed product indices after purchase |
| cart[] | Output | Items currently in cart during browse/checkout |
| cart_full | Output | High when pro_count >= 11 |

Source: vending.v  |  FSM type: Mealy/Moore hybrid  |  State register: synchronous, Reset: asynchronous active-low